What is access point (AP)?

An access point (AP) is a networking device that connects wireless devices to a wired network. It acts as a bridge between the wireless and wired networks, allowing wireless devices to communicate with each other and with wired devices on the same network. APs can be standalone devices or built into a router, and are an important part of many networks due to their ability to provide wireless connectivity while maintaining the reliability and security of a wired connection.

NIC stands for? And at which layer does it exist in the OSI model?

NIC stands for Network Interface Card. It is a hardware component that connects a computer to a network and allows it to communicate with other devices on the network. A NIC exists at the physical layer (Layer 1) of the OSI (Open Systems Interconnection) model, which is responsible for the actual transmission and reception of data over a physical medium. A NIC can be a separate expansion card that is installed in a computer, or it can be integrated into the motherboard or other components of the computer. It typically has a connector for attaching a network cable, as well as a unique identifier, such as a MAC (Media Access Control) address, that is used to identify the device on the network.

Write the name of device that is used to connect similar types of networks?

A router is a device that connects two or more networks (LANs) together. A router uses address information to route the data packets it receives to the correct locations.

On which layer of the OSI model does a WAN technology exist?

WAN (Wide Area Network) technologies typically operate at the OSI model's Layer 3 (Network Layer) and Layer 2 (Data Link Layer).

Top of Form

What is size of IPv4 address? Also write proper example? And OSI layer?

The size of an IPv4 address is 32 bits. An example of an IPv4 address is 192.168.1.1. It operates at the OSI model's Layer 3 (Network Layer).

Top of Form

Q6: What is size of MAC address? Also write proper example? And OSI layer? (3)

The size of a MAC address is typically 48 bits (6 bytes). An example of a MAC address is 00:1A:2B:3C:4D:5E. It operates at the OSI model's Layer 2 (Data Link Layer).

Q7: Write Difference in between “cat” and “nano” command of Ubuntu? (2)

"cat" is used for displaying or concatenating file contents, "nano" is used for editing files directly from the command line.

Q8: Write Difference in between “ls -l” and “ls -la” command of Ubuntu? (2)

"ls -l" displays detailed information about files and directories excluding hidden ones, while "ls -la" displays detailed information including hidden files.

Top of Form

Q9: Write proper command to edit file on terminal? (2)

nano filename

This command will open the specified file in the Nano text editor, allowing you to make changes to its contents. If the file doesn't exist, Nano will create a new file with that name.

Q10: Write proper command to create 5 folders together named “f1, f2, f3, f4 and f5”. (2)

mkdir f1 f2 f3 f4 f5

This command will create five folders named "f1", "f2", "f3", "f4", and "f5" in the current directory.

Top of Form

Q1: Define network layer of OSI model and its protocol? (5)

The Network Layer (Layer 3) of the OSI (Open Systems Interconnection) model is responsible for the routing of data packets from the source to the destination across multiple networks. Its primary functions include logical addressing, routing, and path determination.

Protocol associated with the Network Layer:

- Internet Protocol (IP): This is the primary protocol used in the Network Layer. It provides logical addressing (IPv4 or IPv6) for devices on the network and facilitates the routing of packets between different networks. IP is responsible for breaking down data into packets, addressing them, and routing them to their destinations.

Top of Form

Q2: Write whole Procedure step by step to capture user login in http packets? (5)

- Install Wireshark.

- Open Wireshark and select network interface.

- Start capturing packets.

- Apply HTTP filter.

- Initiate login process on target device.

- Look for HTTP packets containing login information.

- Stop packet capture.

- Review captured data.

- Handle credentials securely.